OpenRISC 1000

System Architecture Manual

18/Apr/2000, DRAFT

# About this Manual

## Brief Introduction

OpenRISC 1000 system architecture manual defines architecture for a family of open source, synthesizable RISC microprocessor cores. As an architecture, OpenRISC 1000 allows for a spectrum of chip and system implementations at a variety of price/performance points for a range of applications. It is a 32-bit load and store RISC architecture designed with emphasis on speed, simplicity and scalability. OpenRISC 1000 targets medium and high performance embedded environments.

Architecture itself covers instruction set, register set, cache management and coherency, memory model, exception model, addressing modes, operands conventions and application binary interface (ABI).

This manual does not specify implementation specific details such as pipeline depth, cache organization, branch prediction, instruction timing, bus interface etc.

## Authors

If you have contributed to this manual and your name isn't listed here, it is not meant as a slight.  We just don't know about it.  Send email to the maintainer(s), and we'll correct the situation.

| Name | E-mail | contribution |
| --- | --- | --- |
| Damjan Lampret | lampret@opencores.org | Initial document |

Figure 1-1. Authors of This Manual

## Revision History

Revision history of this manual.

| revision date | BY | MODIFICATIONS |
| --- | --- | --- |
| 15/Mar/2000 | Damjan Lampret | Initial document |

Figure 1-2. Revision History

## Work in Progress

This document is work in progress. Latest version is always available from OPENCORES CVS. See details how to get it on http://www.opencores.org/.

We are currently looking for people working on this document and for a maintainer of this document. If you would like to contribute send an email to one of the authors.

## Fonts in this manual

In this manual, fonts are used as follows:

- Typewriter font is used for programming examples

- Bold font is used for emphasis

- UPPER CASE items may be either acronyms or register mode fields that can be written by software. Some common acronyms appear in the glossary in this chapter

- Square brackets [ ] indicate an addressed field in a register or a numbered register in a register file

# Architecture Overview

This chapter introduces OpenRISC 1000 architecture and describes general architecture features.

## Features

OpenRISC 1000 architecture includes the following principal features:

- A completely open and free architecture

- A linear, 32-bit logical address space with implementation specific physical address space

- Simple and uniform-length instruction formats featuring two different ISAs:

- OR32 Instruction Set for 32 bits wide instructions aligned on 32-bit boundaries in memory operating on 32 bits and 64 bits data

- OR16 Instruction Set for 16 bits wide instructions aligned on 16-bit boundaries in memory operating on 32 bits data

- Simple memory addressing mode where memory address is calculated with addition of register operand and signed 16-bit immediate

- Most instructions operate on two register operands (or one register and a constant), and place the result in a third register

- 32-entry or narrow 16-entry general purpose register file

- Branch delay slot for keeping pipeline as full as possible

- Support for separate instruction and data caches/MMUs (Harvard architecture) or for unified instruction and data caches/MMUs (Stanford architecture)

- A flexible architecture definition that allows certain functions to be performed in either hardware or with assistance of implementation-specific software

- Two external exception (interrupt) and context switch types – fast and superfast

## Introduction

OpenRISC 1000 architecture is completely open architecture. It defines architecture of a family of open source, RISC microprocessor cores. As an architecture, OpenRISC 1000 allows for a spectrum of chip and system implementations at a variety of price/performance points for a range of applications. It is a 32-bit load and store RISC architecture designed with emphasis on speed, simplicity and scalability. OpenRISC 1000 targets mid performance embedded environments.

Performance features include fully 32-bit architecture, powerful virtual memory support, cache coherency, optional SMP and SMT support and support for fast context switching. Architecture defines several features for embedded environments. Most notable are two instruction sets, one with 32-bit and the other with 16-bit instruction sizes, instruction set optimized for embedded environments, configurable number of general purpose registers, configurable cache and TLB sizes, dynamic power management support and space for user provided instructions.

OpenRISC 1000 architecture is a predecessor of more powerful and richful next generation OpenRISC architectures.

Implementions of the OpenRISC 1000 architecture are available in full source from www.opencores.org and are supported with GNU software development tools and with  a behavioral simulator. Most OpenRISC implementations are designed modular and vendor independent. They can be interfaced with other open source cores available from www.opencores.org.

Opencores.org encourages third parties to design and market their own implementations of the OpenRISC 1000 architecture.

## Acronyms and Abbreviations

| ALU | Arithmetic logic unit |
| --- | --- |
| BAT | Block address translation |
| BIU | Bus interface unit |
| BTC | Branch target cache |
| CPU | Central processing unit |
| EA | Effective address |
| FPU | Floating-point unit |
| GPR | General purpose register |
| MMU | Memory management unit |
| PTE | Page table entry |
| R/W | Read/Write |
| RISC | Reduced instruction set computing |
| SMP | Symetrical multi-processing |
| SMT | Simultaneus multi-threading |
| SPR | Special purpose register |
| TLB | Translation lookaside buffer |

Table 2-1. Acronyms and Abbreviations

## Conventions

| h.mnemonic | Identifies OR16 instruction where instruction is 16 bits wide. |
| --- | --- |
| l.mnemonic | Identifies OR32 instruction where instruction is 32 bits wide. |
| 0x | Prefix indicates a hexadecimal number. |
| rA | Instruction syntax used to identify a general purpose register |
| REG[FIELD] | Syntax used to identify specific bit(s) of a general or special purpose register. FIELD can be a name of a one or a group of bits or a numerical range constructed from two values separated by a colon. |
| x | In certain contexts this indicates a don't care. |
| n | In certain contexts this indicates an undefined numerical value. |
| Implementation | Actual processor implementing OpenRISC 1000 architecture. |
| Module | Sometimes refered to as a coprocessor. A unit in implementation usually with some special registers and controlling instructions. It can be defined by the architecture or it can be custom. |
| Exception | A vectored transfer of control to supervisor software through a exception vector table. A way how a processor can request operating system assistance (division by zero, TLB miss, external interrupt etc). |
| Privileged | An instruction (or register) that can only be executed (or accessed) when the processor is in supervisor mode (when SR[SUPV]=1). |

Table 2-2. Conventions

## Numbering

All numbers are decimal or hexadecimal unless otherwise indicated. The prefix 0x indicates hexadecimal number. Decimal numbers don't have any special prefix. Binary and other numbers are marked with their base.

# Addressing Modes and Operand Conventions

This chapter describes memory addressing modes and memory operand conventions defined by OpenRISC 1000 system architecture.

## Memory Addressing Modes

An effective address is computed by the processor when executing memory access or branch instruction or when fetching the next sequential instruction. If the sum of the effective address and the operand length exceeds the maximum effective address in logical address space, the memory operand is considered to wrap around from the maximum effective through effective address 0.

### Register Indirect with Displacement

Load/store instructions using this address mode contain a signed 16-bit immediate which is sign extended and added to the contents of a general-purpose register specified in the instruction.

Figure 3-3. Register Indirect with Displacement Addressing

Figure 3-3 shows how an effective address is computed when using register indirect with displacement addressing mode.

### Link Register Indirect

Jump instructions using this address mode use the contents of the LR as an effective address.

Figure 3-4. Link Register Indirect Addressing

Figure 3-4 shows how an effective address is generated when using link register indirect addressing mode.

### PC Relative

Branch instructions using this address mode contain a signed 26-bit immediate which is sign extended and added to the contents of a Program Counter register.

Figure 3-5. PC Relative Addressing

Figure 3-5 shows how an effective address is generated when using PC relative addressing mode.

## Memory Operand Conventions

The architecture defines an 8-bit byte, 16-bit halfword, a 32-bit word and a 64-bit doubleword.

| Term | length in Bytes | length in Bits |
| --- | --- | --- |
| byte | 1 | 8 |
| halfword (or half) | 2 | 16 |
| singleword (or word) | 4 | 32 |
| doubleword (or double) | 8 | 64 |

Table 3-3.  Memory Operands

### Bit and Byte Ordering

Byte ordering defines how the bytes that make up halfwords, words, doublewords are ordered in memory. To simplify OpenRISC implementations architecture specifies as default byte ordering the most significant byte (MSB) ordering, or big endian as it is sometimes called. But implementation can support least significant byte (LSB) ordering if they implement byte reording hardware. Reordering is enabled with bit SR[LEE].

The figures below illustrate the conventions for bit and byte numbering within various width storage units. These conventions hold for both integer data and floating-point data, where the most significant byte of a floating-point value holds the sign and at least the start of the exponent.

Table 3-4 shows how bits and bytes are ordered in a halfword.

| Bit 15 |  | Bit 8 | Bit 7 |  | Bit 0 |
| --- | --- | --- | --- | --- | --- |
| MSB |  |  | LSB |  |  |
| Byte address 0 |  |  | Byte address 1 |  |  |

Table 3-4. Default Bit and Byte Ordering in Halfwords

Table 3-5 shows how bits and bytes are ordered in a singleword.

| Bit 31 |  | Bit 24 |  |  | Bit 7 |  | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MSB |  |  |  |  | LSB |  |  |
| Byte address 0 |  |  | Byte address 1 | Byte address 2 | Byte address 3 |  |  |

Table 3-5. Default Bit and Byte Ordering in Singlewords

Table 3-6 shows how bits and bytes are ordered in a doubleword.

| Bit 63 |  | Bit 56 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MSB |  |  |  |  |  |  |  |
| Byte address 0 |  |  | Byte address 1 | Byte address 2 | Byte address 3 |  |  |
|  |  |  |  |  | Bit 7 |  | Bit 0 |
|  |  |  |  |  | LSB |  |  |
| Byte address 4 |  |  | Byte address 5 | Byte address 6 | Byte address 7 |  |  |

Table 3-6. Default Bit and Byte Ordering in Doublewords

### Alignment and Misaligned Accesses

A memory operand is naturally aligned if its address is integral multiple of the operand length. Implementation might support accessing unaligned memory operands but default behavioral is that accesses to unaligned operands result in alignment exception. See chapter “Exception Model” on page 23 for information on alignment exception.

| Operand | Length | addr[3:0] if aligned |
| --- | --- | --- |
| Byte | 8 bits | xxxx |
| Halfword (or half) | 2 bytes | xxx0 |
| Singleword (or word) | 4 bytes | xx00 |
| Doubleword (or double) | 8 bytes | x000 |

Table 3-7. Memory Operand Alignment

OR32 instructions are four bytes long and word-aligned. OR16 instructions are two or more bytes long and always halfword-aligned.

# Register Set

## Features

OpenRISC 1000 register set includes the following principal features:

- thirtytwo or sixteen 32-bit general-purpose registers – OpenRISC 1000 implementations optimized for use in FPGAs and ASICs in embedded and similar environments might use only the first sixteen of all thirtytwo registers.

- thirtytwo 64-bit floating-point registers.

- all other registers are special-purpose registers defined for each module separately and accessible through mtsr/mfsr instruction pair

## Overview

An OpenRISC 1000 processor includes several types of registers: general-purpose and special-purpose user-level registers, system control/status registers and module dependent registers.

General-purpose and special-purpose user-level registers are accessible both in user mode and supervisor mode of operation. System control registers are accessible only in supervisor mode of operation (SR[SUPV]=1).

Module dependent registers are usually accessible only in supervisor mode but not necessary. For architecture-defined modules accessibility is defined in this manual. For custom modules not covered by this manual accessibility is defined in documentation accompanying those modules.

## Special Sixteen GPRs Support

Programs can be compiled with upper sixteen registers set as fixed registers. Such programs are also executable on normal implementation with thirtytwo registers but not vice versa. This feature is quite useful since customers will move from less powerful OpenRISC implementations with sixteen registers to more powerful thirtytwo register OpenRISC implementations.

It is also possible to run code compiled for thirtytwo register implementation on OpenRISC implementations with just sixteen registers since in this case all instructions that access upper sixteen registers are traped with register range exception. Emulation of upper sixteen registers is left to the register range exception handler routine and to implementation specific hardware support.

Figure 4-6. OpenRISC 1000 Programming Model - Registers

## User Level Registers

### General-Purpose Registers (GPRs)

The thirtytwo 32-bit general-purpose registers are labeled R0-R31. They hold integer data or memory pointers used by instructions. Table 4-8 contains a list of general-purpose registers and functions for which they are used. The GPRs are accessed as source and destination registers in the instruction syntax.

| Register |  |  |  |  | r31 | r30 |
| --- | --- | --- | --- | --- | --- | --- |
| Function |  |  |  |  | SAV10 | TMP10 |
| Register | R29 | R28 | r27 | r26 | r25 | r24 |
| Function | SAV9 | TMP9 | SAV8 | TMP8 | SAV7 | TMP7 |
| Register | R23 | R22 | r21 | r20 | r19 | r18 |
| Function | SAV6 | TMP6 | SAV5 | TMP5 | SAV4 | TMP4 |
| Register | R17 | R16 | r15 | r14 | r13 | r12 |
| Function | SAV3 | TMP3 | SAV2 | TMP2 | SAV1 | TMP1 |
| Register | R11 | r10 | r9 | r8 | r7 | r6 |
| Function | RVAL | SAV0 | TMP0 | LR | ARG4 | ARG3 |
| Register | R5 | r4 | r3 | r2 | r1 | r0 |
| Function | ARG2 | ARG1 | ARG0 | FP | SP | Zero |

Table 4-8. Lower and Upper Parts of General-Purpose Registers

R0 is used as a constant zero. Whether is R0 actually hardwired to zero is implementation dependent. R0 should never be used as a destination register. Functions of other registers are explained in chapter “Application Binary Interface” on page 30.

Implementation may have several sets of GPRs and use them as shadow registers, switching between them whenever a new exception occurs. Current set is identified with SR[CID] value.

Implementation is not required to initialize GPRs to zero during reset procedure. It is a responsibility of a reset exception handler to initialize GPRs to zero if that is necessary.

### Floating-Point Registers (FPRs)

The thirtytwo floating-point registers are 64 bits wide and labeled FPR0–FPR31. Table 4-9 contains a list of these floating-point registers. The FPRs are accessed as source and destination registers in the floating-point instructions. See chapter “Application Binary Interface” on page 30 for information on floating-point data types.

| Register |  |  | FPR31 | FPR30 | FPR29 | FPR28 |
| --- | --- | --- | --- | --- | --- | --- |
| REGISTER | FPR27 | FPR26 | FPR25 | FPR24 | FPR23 | FPR22 |
| Register | FPR21 | FPR20 | FPR19 | FPR18 | FPR17 | FPR16 |
| Register | FPR15 | FPR15 | FPR15 | FPR14 | FPR13 | FPR12 |
| REGISTER | FPR11 | FPR10 | FPR9 | FPR8 | FPR7 | FPR6 |
| Register | FPR5 | FPR4 | FPR3 | FPR2 | FPR1 | FPR0 |

Table 4-9. Floating-Point Registers

### Condition Code Register (CCR)

Condition code register is a 32-bit special-purpose user-level register accessible with mtsr/mfsr instruction pair.

Flag named FLAG is set by sfXX instructions as a result of a compare operation. Flag named CARRY is set by arithmetic operations as a result of a carry out and used with addic instruction. Flag named OVERFL is set by arithmetic operations when overflow occurs.

| Bit | 31-3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- |
| Identifier | Reserved | OVERFL | CARRY | FLAG |
| Reset | 0 | 0 | 0 | 0 |
| R/W | Read Only | R/W | R/W | R/W |

| FLAG | Conditional branch flag 0 FLAG flag was cleared by sfXX instructions 1 FLAG flag was set by sfXX instructions |
| --- | --- |
| CARRY | Carry flag 0 No carry out produced by last arithmetic operation 1 Carry out was produced by last arithmetic operation |
| OVERFL | Overflow flag 0 No overflow occured during last arithmetic operation 1 Overflow occured during last arithmetic operation (might even result in a overflow exception) |

Table 4-10. CCR Field Descriptions

### Link Register (LR) (obsolete; will be removed)

The link register is a special-purpose user-level register accessible with mtsr/mfsr instruction pair. It has a width of logical addresses and in OpenRISC implementations with 32-bit logical address space is a 32-bit register.

It supplies the logical branch target address for bflr/bnflr/jlr instructions and is used to hold the logical address of instruction that follows a jal instruction. Lower two bits are ignored.

### Count Register (CTR) (obsolete; will be removed)

The count register is a special-purpose user-level register accessible with mtsr/mfsr instruction pair. It has a width of logical addresses and in OpenRISC implementations with 32-bit logical address space is a 32-bit register.

It holds a loop count that can be decremented during execution of sfXX instructions with appropriately coded CNT field. When the condition encoded in CNT field of sfXX instruction is meet then a FLAG flag is set.

## Supervisor Level Registers

### Supervision Register (SR)

The supervison register is a 32-bit special-purpose supervisor-level register accessible with mtsr/mfsr instruction pair only in supervisor mode.

It defines the state of the processor.

| Bit | 31-28 | 27-8 | 7 |
| --- | --- | --- | --- |
| Identifier | CID | Reserved | LEE |
| Reset | 0 | 0 | 0 |
| R/W | R/W | Read Only | R/W |

| Bit | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Identifier | IME | DME | ICE | DCE | EIR | EXR | SUPV |
| Reset | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| R/W | R/W | R/W | R/W | R/W | R/W | R/W | R/W |

| SUPV | Supervisor Mode 0 Processor is in User Mode 1 Processor is in Supervisor Mode |
| --- | --- |
| EXR | Exception Recognition 0 Exceptions are not recognized 1 Exceptions are recognized |
| EIR | External Interrupt Recognition 0 External Interrupts are not recognized 1 External Interrupts are recognized |
| DCE | Data Cache Enable 0 Data Cache is not enabled 1 Data Cache is enabled |
| ICE | Instruction Cache Enable 0 Instruction Cache is not enabled 1 Instruction Cache is enabled |
| DME | Data MMU Enable 0 Data MMU is not enabled 1 Data MMU is enabled |
| IME | Instruction MMU Enable 0 Instruction MMU is not enabled 1 Instruction MMU is enabled |
| LEE | Little Endian Enable 0 Little Endian (LSB) byte ordering is not enabled 1 Little Endian (LSB) byte ordering is enabled |
| CID | Context ID 0-15 Current Processor Context |

Table 4-11. SR Field Descriptions

### PC Saved Register (PCSR)

### Exception EA Register (EEAR)

### Instruction/Data MMU Modules Registers

### Instruction/Data Cache Modules Registers

### Performance Monitor Module Registers

### Debug Module Registers

### Time Base Module Registers

### Custom Modules

## List of All Special-Purpose Registers

Special-purpose registers of all modules are grouped into sixteen groups. Each group can have different register address decoding depending on a maximum theorethical number of registers in that particular group. One group can contain registers from several different modules. In register address decoding it is also used SR[SUPV] bit since some registers are accessible only in supervisor mode. Instructions for reading and writing registers are mtsr and mfsr.

| group | module description |
| --- | --- |
| 0 | System Control and Status Registers |
| 1 | Data MMU (in case of a single unified MMU groups 1 and 2 decode in a single set of registers) |
| 2 | Instruction MMU (in case of a single unified MMU groups 1 and 2 decode in a single set of registers) |
| 3 | Data Cache (in case of a single unified cache groups 3 and 4 decode in a single set of registers) |
| 4 | Instruction Cache (in case of a single unified cache groups 3 and 4 decode in a single set of registers) |
| 5 | Performance Monitor Unit |
| 6 | Debug Unit |
| 7 | Time Base Unit |
| 8-11 | Reserved for future use |
| 12-15 | Custom modules |

Table 4-12. List of Groups and Modules

OpenRISC 1000 implementation is required to implement at least special-purpose registers from group 0. All others groups are optional and registers from these groups are implemented only if the implementation has a coresponding module. Which modules are implmeneted can be read from MDLCFGR register in group 0.

| Group | Register Number | Register Name | Access | Description |
| --- | --- | --- | --- | --- |
| 0 | 1 | CTR | User/Supervisor | Count Register (obsolete) |
| 0 | 2 | LR | User/Supervisor | Link Register (obsolete) |
| 0 | 3 | PCSR | Supervisor | PC Saved Register |
| 0 | 4 | SR | Supervisor | Supervision Register |
| 0 | 5 | MDLCFGR | Supervisor | Module Configuration Register |
| 0 | 6 | EEAR | Supervisor | Exception EA Register |

Table 4-13. List of All Special-Purpose Registers

# Instruction Set

This chapter describes OR32 and OR16 instruction sets.

## Features

OpenRISC 1000 instruction set includes the following principal features:

- Simple and uniform-length instruction formats featuring two different Instruction Set Architectures (ISA)

- OR32 Instruction Set with 32 bits wide instructions aligned on 32-bit boundaries in memory operating on 32 bits and 64 bits data

- OR16 Instruction Set with 16 bits wide instructions aligned on 16-bit boundaries in memory operating on 32 bits data

- Reserved opcodes for floating-point and vector instructions

- Reserved opcodes for custom instructions

- Instructions divided into instruction classes where only the basic classes are required to be implemented in OpenRISC 1000 implementation

## Overview

OpenRISC 1000 instructions belong to one of the following ISAs:

- OR32:

- integer instructions

- floating-point instructions

- vector instructions

- load and store instructions

- program flow instructions

- module control and supervisor-level instructions

- OR16:

- integer instructions

- load and store instructions

- program flow instructions

- module control and supervisor-level instructions

Instructions in each ISA are also split into instruction classes according to implementation importance:

- user-level basic class

- user-level recommended class

- user-level optional class

- supervisor-level basic class

- supervisor-level optional class

- several module and implementation dependent classes including custom class

- illegal class

| OR32 | OR16 | ISA Class |
| --- | --- | --- |
| Basic load/store, integer, program flow control instructions | Basic load/store, integer, program flow control instructions | User-level basic class |
| Additional highly recommended integer arithmetic instructions, floating-point and vector instructions | Additional highly recommended integer instructions | User-level recommended class |
| Rarely used integer, floating-point or vector instructions | Rarely used integer instructions | User-level optional class |
| Move from/to special registers, rfe | Move from/to special registers, rfe | Supervisor-level basic class |
| Implementation specific instructions defined in architecture for memory management, context switch etc. | Implementation specific instructions defined in architecture for memory management, context switch etc. | Supervisor-level optional class |
| Implementation and module specific instructions defined in implementation documentation including custom instructions | Implementation and module specific instructions defined in implementation documentation including custom instructions | Module and implementation dependent and custom classes |
| Reserved and illegal opcodes | Reserved and illegal opcodes | Illegal class |

Table 5-14. OpenRISC 1000 Instruction Sets and Classes

## OR32

## OR16

# Exception Model

This chapter describes exception mechanism, exception types and their handling.

## Introduction

Exception mechanism allows the processor to change to supervisor state as a result of external signals, errors, or unusual conditions arising in the execution of instructions. When exceptions occur, information about the state of the processor is saved to certain registers and the processor begins execution at the address predetermined for each exception. Processing of exceptions begins in supervisor mode.

OpenRISC 1000 defines special support for fast exception processing also called superfast context switch support. This allows very rapid interrupt processing. It is achieved with shadowing general-purpose and some special registers.

Architecture requires that all exceptions are handled in strict order with respect to the instruction stream. When an instruction-caused exception is recognized, any unexecuted instructions that appear earlier in the instruction stream are required to complete before the exception is taken.

Exceptions can occur while an exception handler routine is executing, and multiple exceptions can become nested. Support for fast exceptions allows fast nesting of exceptions until all shadowed registers are used.

## Exception Classes

All exceptions can be described as precise or imprecise and either synchronous or asynchronous. Synchronous are caused by instructions and asynchronous are caused by events external to the processor.

| Type | Exception |
| --- | --- |
| Asynchronous/nonmaskable | Bus Error Reset |
| Asynchronous/maskable | External Interrupt |
| Synchronous/precise | Instruction-caused exceptions excluding floating-point imprecise exceptions |
| Synchronous/imprecise | Instruction-caused floating-point imprecise exceptions |

Table 6-15. Exception Classes

Current PC is saved to PCSR. If I-TLB, D-TLB miss or one of page fault exceptions EEAR is set with effective address in question. Other registers must be saved by exception handler routine. Processing continues from:

| Exception Type | Vector Offset | causing conditions |
| --- | --- | --- |
| Reset | 0x100 | Caused by soft and hard reset. |
| Bus Error | 0x200 | The causes are implementation-specific, but typically they are related to bus errors and attempts to access invalid physical address. |
| Data Page Fault | 0x300 |  |
| Instruction Page Fault | 0x400 |  |
| External Interrupt | 0x500 |  |
| Alignment | 0x600 |  |
| Illegal Instruction | 0x700 |  |
| Priority External Int | 0x800 | High priority interrupt (superfast context switch) |
|  | 0x900 |  |
|  | 0xA00 |  |
| Register Range | 0xB00 | Only on OpenRISC implementations with 16 GPRs |
| System Call | 0xC00 | (Superfast context switch) |
| Breakpoint | 0xD00 |  |
|  | 0xE00 |  |
|  | 0xF00 |  |
| Reserved | 0x1000 - 0x1F00 | Reserved for implementation-specific exceptions. |

Table 6-16. Exception Types and causing conditions

## Exception Processing

## Context Switches – Fast and Superfast

# Memory Model

# Memory Management

This chapter describes the virtual memory and access protection in memory management of OpenRISC 1000 architecture.

## MMU Features

OpenRISC 1000 memory management unit includes the following principal features:

- Support for implementation specific size of logical, virtual and physical address spaces

- Support for variable page sizes

- Powerful page based access protection with support for demand-paged virtual memory

- Support for simultaneous multi-threading (SMT)

## MMU Overview

Translation diagram:

PTE:

Each PTE is 4 bytes in size. Enough?

PP Index: index to an array of registers for page protection. Each page protection policy specifies RWX bits for supervisor and for user mode (so 6 bits per policy, 8 possible policies).

U2, U1: used by operating system for what ever reason.

D: dirty (page will have to be written back to swap and not simply discarded from physical memory)

A: accessed (if page is accessed; a hint to OS which pages to replace when new pages must be brought into physical memory)

OO: Out-of-Order accesses (load only? store only? both?)

WB: controls cache operations: write back type of caches else write through caches

CI: controls cache operations: cache inhibit

MC: memory coherency (or should we call it cache coherency)

V: valid

# Cache Model and Memory Coherency

# Application Binary Interface

### Fundamental Types

## Aggregates and Unions

## Register Usage

## Calling Conventions

### Stack Frame

### Argument Passing

### Return Values

## Operating System Interface

## Position-Independent Code

## ELF

### Sections

### Relocation

## A.OUT

### Sections

### Relocation

Copyright (C) 2000 OPENCORES.ORG and Authors

This document is free; you can redistribute it and/or modify it under the terms of the GNU General Public License as published by the Free Software Foundation; either version 2 of the License, or (at your option) any later version.

This document is distributed in the hope that it will be useful, but WITHOUT ANY WARRANTY; without even the implied warranty of MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE.  See the GNU General Public License for more details.